# Cryptography and Network Security

Network Security Using Adaptive Defense Techniques: Behavior-Based Autonomous Defense with a Linux Gateway.

### Prepared by:

- Joelle Kasabreh		ID: 12428112

### Dr. Jehad Hamamreh

Abstract

This research presents a practical adaptive network defense system based on real-time traffic behavior analysis. A Linux machine was configured as a gateway through which network traffic passes. The system monitors TCP SYN packets using tcpdump [3] and analyzes the number of packets sent by each source IP within a defined time window. When a predefined threshold is exceeded, the activity is considered malicious, and the attacker is blocked using iptables [4]. The system was tested using Nmap SYN scanning techniques [2], and results show that it successfully detects and responds to suspicious behavior automatically.

1. Introduction

TCP SYN-based attacks such as port scanning and SYN flooding are widely used in network intrusion attempts. These attacks exploit the TCP three-way handshake process defined in the TCP protocol [7]. Traditional security systems often rely on predefined rules, while modern approaches focus on analyzing traffic behavior dynamically [10][11]. This project implements a simple adaptive defense mechanism that detects abnormal SYN packet patterns in real time using Linux networking tools.

2. Problem Statement

Small and medium-scale networks often lack lightweight and practical security systems capable of detecting SYN-based attacks without relying on complex intrusion detection systems. Additionally, signature-based approaches are limited in detecting new or slightly modified attack patterns [10]. Therefore, the goal of this work is to develop a system that:

- Monitors traffic passing through a Linux gateway

- Detects abnormal SYN packet behavior

- Automatically blocks the attacker

3. Literature Review

TCP behavior is formally defined in RFC 793 [7], while SYN flood attacks and mitigation techniques are discussed in RFC 4987 [8]. Nmap provides a widely used implementation of SYN-based port scanning [2]. Tools such as tcpdump are used for packet-level monitoring [3], while iptables is used to control and filter network traffic in Linux environments [4]. Previous research highlights that behavior-based detection methods can effectively identify anomalies in network traffic patterns [10][11].

4. Proposed Approach

The proposed system is based on a behavior-driven detection model. It operates as follows:

- A Linux machine acts as the network gateway

- Network traffic is monitored using tcpdump [3]

- SYN packets are counted per source IP

- A time window and threshold are applied

- If the threshold is exceeded → attack is detected

- The attacker is blocked using iptables [4]

The system focuses entirely on traffic behavior rather than static signatures.

5. Methodology

## 5.1 Network Setup

The experimental environment consists of:

- A Linux machine configured as a gateway

- A client device connected through a wireless access point

- A router configured as an Access Point (DHCP disabled)

All client traffic is directed through the Linux gateway.

## 5.2 Enabling IP Forwarding

sysctl -w net.ipv4.ip_forward=1

This enables packet forwarding within the Linux system [5].

## 5.3 NAT Configuration

iptables -t nat -A POSTROUTING -o <interface> -j MASQUERADE

## 5.4 Traffic Monitoring

TCP SYN packets are monitored using tcpdump:

tcpdump -i any tcp[tcpflags] & tcp-syn != 0 and tcp[tcpflags] & tcp-ack == 0

## 5.5 Behavior Analysis

A Python script was implemented to:

- Count SYN packets per IP

- Maintain a time window (e.g., 10 seconds)

- Apply a threshold (e.g., 5 packets)

- Trigger detection when the threshold is exceeded

## 5.6 Attacker Identification

The attacker’s MAC address is obtained using:

ip neigh show <ip>

## 5.7 Blocking Mechanism

Traffic from the attacker is blocked using iptables:

iptables -I FORWARD -s <ip> -j DROP

iptables -I INPUT -s <ip> -j DROP

If available, MAC-based blocking is also applied.

6. System Implementation

The system is composed of the following components:

- Monitoring Module: Captures SYN packets using tcpdump

- Analysis Module: Python script processes packet counts

- Blocking Module: iptables rules enforce access control

- Logging Module: Records events and blocked IPs

These components operate together to detect and mitigate attacks.

7. Results and Discussion

The system was tested using the following command:

nmap -sS -T5 --min-rate 20000 -p- <target>

## Observed Results:

- Continuous SYN packet generation was detected

- Packet count increased per source IP

- Threshold was exceeded within the defined time window

- The system detected the attack

- The attacker’s IP was blocked

- Events were logged successfully

These results align with expected SYN-based attack behavior described in [8].

# 8. Advantages of the System

- Utilizes standard Linux tools without requiring additional frameworks

- Operates in real time

- Does not rely on predefined signature databases

- Can be extended or modified easily

- Implements behavior-based detection techniques

# 9. Limitations

- Requires traffic to pass through the Linux gateway

- Dependent on correct network configuration

- Fixed threshold may lead to false positives in high-traffic environments

- Limited to SYN-based attack detection

# 10. Conclusion

This project demonstrates a practical implementation of an adaptive network defense system using Linux tools. The system successfully monitors traffic, detects abnormal SYN packet behavior, and blocks attackers automatically. It provides a simple yet effective approach to protecting networks against SYN-based attacks and can be enhanced with more advanced detection techniques in future work.

# References

[1] Fyodor, Nmap Network Scanning, https://nmap.org/book/
[2] Nmap SYN Scan Documentation, https://nmap.org/book/synscan.html
[3] tcpdump Manual, https://man7.org/linux/man-pages/man8/tcpdump.8.html
[4] iptables Manual, https://man7.org/linux/man-pages/man8/iptables.8.html
[5] Linux IP Forwarding Documentation, https://www.kernel.org/doc/Documentation/networking/ip-sysctl.txt
[6] Python subprocess Documentation, https://docs.python.org/3/library/subprocess.html
[7] RFC 793 – Transmission Control Protocol, https://datatracker.ietf.org/doc/html/rfc793
[8] RFC 4987 – TCP SYN Flooding Attacks, https://datatracker.ietf.org/doc/html/rfc4987
[9] Linux Networking Guide, https://tldp.org/HOWTO/NET3-4-HOWTO.html
[10] Stallings, Network Security Essentials, Pearson
[11] Kurose & Ross, Computer Networking: A Top-Down Approach
[12] Behl & Behl, Cybersecurity and Cyberwar, Oxford